| Name: | Dhuware Chaitanya Rakesh |
| --- | --- |
| Roll Number: | 20IM10009 |

1.Aim of the experiment:

- Familiarisation with Signal Generator, Oscilloscope

- Studies on RC, CR and RL circuits

2.Tools used:

- VS Labs

- Falstad

3.Background knowledge (brief):

- Voltage Divider (from Expt1)

- A voltage divider is a simple circuit which turns a large voltage into a smaller one. Using just two series resistors and an input voltage, we can create an output voltage that is a fraction of the input. A voltage divider involves applying a voltage source across a series of two resistors.

- For a voltage divider: Vout = Vin(R2/R1+R2).

- In Thevenin’s equivalent circuit of the voltage divider, the voltage source is replaced by an equivalent Thevenin’s voltage and all the resistors are replaced by a single equivalent Thevenin’s resistance.

- Voltage Divider (Expt2)

- The main purpose of an oscilloscope is to graph an electrical signal as it varies over time. Most scopes produce a two-dimensional graph with time on the x-axis and voltage on the y-axis.

- Uses:

► Measurement of peak-to-peak voltage of a waveform

► Frequency of periodic signals and time between pulses

► Observing constant time varying signal voltages

► Shape of input/output signa ls in a circuit

- Frequency Response

- Frequency Response of an electric or electronics circuit allows us to see exactly how the output gain (known as the magnitude response) and the phase (known as the phase response) changes at a particular single frequency, or over a whole range of different frequencies.

- The -3dB corner frequency points define the frequency at which the output gain is reduced to 70.71% of its maximum value. The -3dB point is also the frequency at which the systems gain has reduced to 0.707 of its maximum value. The -3dB point is also known as the half-power points since the output power at this corner frequencies will be half that of its maximum 0dB.

- Frequency Response of low pass RC/RL filter is nearly flat for low frequencies and all of the input signal is passed directly to the output, resulting in a gain of nearly 1, called unity, until it reaches its Cut-off Frequency point (fc). After this cut-off frequency point the response of the circuit decreases to zero at a slope of -20dB/ Decade or (-6dB/Octave) “roll-off”

- The Frequency Response Curve above for a passive high RC/RL pass filter is the exact opposite to that of a low pass filter. Here the signal is attenuated or damped at low frequencies with the output increasing at +20dB/Decade (6dB/Octave) until the frequency reaches the cut-off point (fc)

- The formula to find the frequency cut-off point of an RC circuit is, frequency= fc =1/2πRC.

- The formula to find the frequency cut-off point of an RC circuit is, frequency= fc = R/(2πL)

- Pulse Response

- pulse response refers to the reaction of any dynamic system in response to some external change.

The Following Setup is being made:

4.Circuit (hand drawn/image)

PULSE RESPONSE

5.Measurement Data (Tabular form)

| SRNo | RL | XL | VL(oscilloscope) | Calculated VL |
| --- | --- | --- | --- | --- |
| 1 | infinity | infinity | 2 | 1.78 |
| 2 | 2.15 | 2.15 | 1.25 | 1.123 |
| 3 | 1 | 1 | 0.83 | 0.77 |
| 4 | 0.6825 | 0.6825 | 0.63 | 0.7 |

Low Pass Filter

High Pass Filter

6.Graph (Image)/Screenshots

7.Conclusion

Voltage Divider(expt1)

Vout=Vin(R2/R1+R2)

= 10(20/20+10)

= 6.67 V which can be seen in the voltage divider circuit diagram.

RC HPF

From graph the cutoff frequency comes out to be ~8111Hz

While calculated frequency is ~7922Hz

Error = 2.4%

RC LPF

From graph the cutoff frequency comes out to be ~1520KHz

While calculated frequency is ~1592KHz

Error = 4.5%

8.Discussions

- A RC LPF acts as an integrator. It integrates the input signal into triangular   waveforms(ideal).

- A RC HPF acts as a differentiator. It differentiates the input signal into frequency spikes. Vo decreases as capacitor charges

- A RL HPF acts as a differentiator also. It differentiates the input signal into frequency spikes. Vo decreases as steady stage is reached.

- The -3dB point is also known as the half-power points since the output power at this corner frequencies will be half that of its maximum 0dB value as shown above

- These -3dB corner frequency points define the frequency at which the output gain is reduced to 70.71% of its maximum value. Then we can correctly say that the -3dB point is also the frequency at which the systems gain has reduced to 0.707 of its maximum value.